Yagi Analysis Ver. 3.54

INTRODUCTION

Yagi Analysis is a Yagi calculating program that has been on the

market for a couple of years now.

Ver 3.0 has been  restyled and now includes both drop-

down and pop-up menus.

Mouse support is also included.

Calculations can be based on one or up to 10 frequencies.

When calculating an antenna using several frequencies a comparison

of radiation pattern and flow for the various frequencies is made.

Yagi Analysis also calculates the antenna's efficiency.

With Ver 3.52  you can calculate G/T and stack up to 44 element Yagis.

You can  save your antennafiles in NEC format, and if you have NEC

you can compare your antenna pattern with the pattern calculated with

NEC.

In ver 3.52 the Gain  has been reduced  with the loss.

Some improvements have been made in Ver 3.52 with the end effect

calculations  on large element diameters.

RELIABILITY

Yagi Analysis is of the same class as NEC for normal Yagi

antennas.

A comparison has shown that results from Yagi analysis have agreed

with NEC.

Some improvements have been made in Ver 3.52 with the end effect

calculations  on large element diameters.

On the other hand, these results have not agreed with other

antenna programs such as Yagimax, MN, YO and other Mininec based

programs. This is because these programs do not agree very well

with NEC, even if this claim is sometimes made.

Yagi Analysis is 5-100 times faster than NEC depending on antenna

size.

The time gain increases with the size of the antenna.

INSTALLATION

To be able to use the Yagi Analysis program an IBM PC or

compatible computer is needed with more than 500K of free memory.

The computer must also be equipped with a hard disk.

The program supports CGA, EGA and HGC.

An Epson compatible printer is needed to print out the screen

graphics.

Using a maths co-processor is recommended to increase speed of

calculation.

If CGA is being used, GRAFTABL and GRAPHICS must be loaded before

running Yagi Analysis. The programs can be found on your DOS disk

and are used when printing screen graphics.

1. Insert program disk  into drive A: and

Type A:

Press <RETURN>

Type INSTALL

Press <RETURN>

This starts the Install program. You must now enter  drive and

path where you want to have Yagi Analysis .

Default is:   C:\YAGI

You must also enter Drive and Path for NEC out and data files.

Default is:   C:\NEC2   and   C:\NEC2\DATA

Press F9 to continue.

Now you can enter the type of graphics card you are using.

You can choose between EGA, CGA and Hercules.

After making your choice you must press < RETURN >

Install will now copy Yagi Analysis to the harddisk and  ask you

for the serial number that is on the disk label.

After typing the serial number press <RETURN> and you will then be

asked for your name, signal or the name of the firm that owns the

program.

It is important that you initiate the disk properly to enable you

to upgrade the program at reduced cost as new versions are

released.

Press <RETURN>.

The program has now been initiated and is ready to use.

Starting YAGI ANALYSIS

Type: YAGI

Press <RETURN>

The program will now start and an information text will appear on

the screen, showing the program name and serial number.

Press <RETURN> to continue.

PROGRAM DESCRIPTION

Menus

The program has drop-down and pop-up menus. It also supports a

Microsoft mouse.

At the top of the screen is the main menu:

FILES  EDIT  RUN  RESULTS  OPTIONS

The current date is also shown at the top of the screen.

If an antenna file has been loaded the file name will be displayed

between the menu and current date.

At the bottom of the screen the program name, version number and

your name will be displayed.

You can access the drop-down menu by pressing <ALT> and the letter

that is highlighted when you press <ALT>.

If you have a mouse the menus can be accessed by clicking on the

menu alternative you want.

After pulling down a menu the options can be selected in three

different ways.

1. Each line of the menu has a letter that is highlighted in a

different colour. By pressing the corresponding letter

on yhe keyboard the option will be activated.

2. An option can also be selected by pressing the UP and DOWN

arrows on the keyboard and pressing <RETURN>.

3. If you are using a mouse just position the pointer on the

option you want and click the mouse button.

To exit from an option press <ESC> or click on the <CANCEL>

option.

If you have changed some data or entered new data in a sub-menu

this must be confirmed by moving the cursor with the <TAB> key to

<OK> and pressing <RETURN>.

You can also use your mouse by clicking on <OK>.

If you press <ESC> or click on <CANCEL> the data you have entered

will be ignored.

You can reach the Help text by pressing F1

A SHORT DESCRIPTION OF THE MENUS.

The first menu alternative is called Files and includes the

following options or sub-menus:

New

Load

Save

Save Nec

Print

Plot

Exit

New is for entering new antenna data.

Load is for retrieving previously entered antenna data.

Save is for storing new data or an antenna calculation.

Save Nec is for saving data in Nec format.

Print is for printing out antenna data after calculation.

Plot is for plotting the radiation pattern in polar form on an HP-

7475 plotter.

Exit is for exiting the program.

EDIT menu

This menu includes the following options or sub-menus:

Elementdata

Length of Taper

Radii of Taper

Frequency

Director number

Boom corr

Elementdata is for entering and changing element position,

spacing,lengths and radii.

Length of Taper is for entering and changing length of element

taper.

Radii of Taper is for entering and changing radii of element taper

Frequency is for changing the starting frequency, frequency

interval and number of frequencies to be analysed.

Director number is for changing the number of directors on an

antenna.

Boom corr is for correcting element lengths to compensate for the

effect the boom has on the antenna.

RUN menu

This menu has only one option - "Run", and is used when making

antenna calculations.

RESULTS menu

This menu is for viewing the results of a calculation and consists

of the following sub-menus:

Data

Polar E-Pattern

Polar H-Pattern

Rectangular E-Pattern

Rectangular H-Pattern

Result-Graf

Current-Graf

Current-Phase

Current

Showing Noise Temperatue (G/T)

Data shows a summary of the results of the calculation, e.g. Gain

Imp, F/B etc.

Polar E- and H-Pattern show the radiation pattern in polar form.

Rectangular E- and H-Pattern show the radiation pattern in

rectangular form.

Result-Graf shows Gain, F/B and VSWR as a function of the

frequency.

Current-Graf shows the relative magnitude of the element currents

in relation to the greatest magnitude.

Current-Phase shows the element phases in a column chart

Current shows the element currents and their phase position and

magnitude.

Showing Noise Temperatue (G/T) shows the Noise Temperature and G/T

computed . (See Options menu)

OPTIONS menu

This menu has the following options or sub-menus:

Dimension unit

Reflector Type

Driver Type

Element Pos

Change Colors

Wire Conductivity

Ground/Stacking

Stacking

Noise Temperatue (G/T)

Dimension unit is for switching between millimetres, inches and

wavelengths.

Reflector Type is for switching the reflector configuration

between single, double, triple and triangular.

Driver Type is for switching between straight and folded dipole.

Element pos is for switching the form in which element spacing

will be entered.

Change Colors is for switching color standard. The options are

Color, Mono and LCD.

Wire Conductivity is for changing the conductivity according to

which material is used in the element. This is necessary in order

to calculate the correct efficiency.

Ground/Stacking is for modelling  the Yagi over a prefectly-

conducting groundplane, or stacking two Yagis.

Stacking is for entering stack spacing and number of antennas when

stacking.

Noise Temperature is for calculating how much Noise the Antenna

will pickup.

A DETAILED DECSCRIPTION OF THE MENUS.

NEW

To analyse a new antenna its data must be entered. To do this the

NEW option is used.

Activate the option by pressing <ALT> + F + N. The menu for

entering new antenna data will now appear.

A number of data fields and a box with default data will now be

displayed. The deafult data box contains Dimension, Refl.Type,

Driver Type and Element pos. You can change these data with the

F5 to F8 tangents.

The first edit field is for the number of directors.

Type a value and press <RETURN>.

Next, you must enter the starting frequency. Type the frequency in

MHz and press <RETURN>.

You must now enter the spacing between each new frequency. Type

the value in MHz and press <RETURN>.

The number of frequencies to be calculated must now be entered.

The highest value allowed is 10. Type a value and press <RETURN>.

The cursor will now be at the STEP: field.

If you have chosen or wish to choose element tapering the number of

tapers per element must be entered here. If you have not chosen

tapering just press <RETURN> and the cursor will move to <OK>.

When you are satisfied with the data you have entered move the

cursor to <OK> and press <RETURN> again to confirm that it is

correct. If you want to change some data you can move to the data

field you wish to change by using the arrow keys. Enter the

correct value and using the <TAB> key move the cursor to <OK>

again.

Press <RETURN>.

Depending on your choice of Tapering or not,( see Element Tapering)

two different menus can appear.

With Tapering element the Length of Taper menu will appear. In

other case the Elementdata menu will appear.

ELEMENT TAPERING

With this option you can select to use element tapering

(telescopic element) or mono-tapering.

Select "Yes" by moving the cursor in the brackets with the

<TAB> key or by clicking on it with the mouse.

With the cursor in the brackets you can select or unselect Element

Tapering by pressing the Spacebar.

You can now move the cursor by pressing <TAB> or clicking with the

mouse on the desired place.

NOTE: Element tapering must be selected before entering new

antenna data. Element tapering cannot be selected if data for a

mono-tapered antenna has already been entered.

When using element tapering Yagi Analysis will recalculate the

elements to mono-tapering with an algorithm from W2PV. This

algorithm converts the telescopic elements to normal non-tapering

elements with the same internal impedance as the telescopic.

When entering the element lengths start from the centre i.e. the

antenna boom, and work towards the tip of the element.

ELEMENTDATA MENU

This menu is for entering the element position, lengths and radii.

It consists of four editfields, four listboxes and five buttons.

The leftmost listbox and editfield is named: EL_name, and

contains the name of the elements.

The other listboxes and editfields contain Element position or

distance between elements, element lengths and element radii.

The five buttons are <OK>, <CANCEL>, <INSERT>, >DELETE>, and

<HELP>.

You can activate these buttons by pressing F5-F1  or by clicking

on a button with the mouse.

EL_pos , EL_Distance

There are two methods for entering the element position.

1. Entering spacing between adjacent elements.

This method is easier to edit if element spacing is changed.

2. Entering the actual position of the element in relation to the

reflector which is assumed to have position "0".

This method is always used with element tapering.

The desired method is chosen at the Options menu.

The position of the first reflector Re1 is always "0".

If more than one reflector is being used (see Options menu) use

the <HELP> option to see how the data is to be entered. The <HELP>

option is activated by pressing F1 or clicking on <HELP> if you

have a mouse.

Enter elementposition ("0" for the first reflector) and press

<RETURN>. The cursor will move to the EL_LENGTH editfield .

By pressing F10 you can lock the cursorto the El_pos editfield.

If chosen method is EL_distance and two or more adjacent elements

have the same spacing between each other, you only need to press

<RETURN> and the previous spacing is copied to the current

editfield.

EL_Length

This editfield is for entering the lengths of the elements.

If two or more adjacent elements have the same length you only

need to press <RETURN> and the previous value is copied to the

current element.

The length must be endpoint-to-endpoint and not

centre-to-endpoint as in some other programs.

If a folded dipole is being used you must go to the Options menu

and select Folded. You will then be asked to enter the dipole

length and diameter.

After pressing <RETURN> the cursor will move to the EL_Radii

editfield.

You can lock the cursor to the EL_length field by pressing F10.

EL_Radii

This editfield is for entering the elementRadii.

Note that it is the radius and not the diameter that is entered.

(See EL_Length for more information)

<INSERT>

Elements can be added by pressing F3 or clicking on <INSERT> if you

have a mouse.

To add an element, first mark the element in front of the place

where the new element is to be added, and press F3.

All data for the new element will be "0".

Enter the new element data and press <RETURN>.

<DELETE>

Elements can be removed by clicking on <DELETE> or by pressing F2.

To remove an element just mark the desired element and press F2.

Elements can be marked by moving the marker line with the arrow key

or by clicking on the desired element if you have a mouse.

If you are satisfired with the data you have entered just press F5

or click on <OK> if you have a mouse.

LENGTH OF TAPER MENU

If you have chosen Tapering element and pressed <OK> in the NEW

menu, the Length of Taper menu will appear.

This menu is for entering the Taper lengths of all elements.

When entering the element lengths start from the centre i.e. the

antenna boom, and work towards the tip of the element.

If two or more adjacent tapers have the same length you only need

to press <RETURN> and the previous value is copied to the current

taper.

You can move the cursor to a desired element tap with the arrow

keys or by pressing the <TAB> key or by clicking on it if you have

a mouse.

If you are satisfied with the data you have entered just activate

the <OK> button with the <TAB> key or (and) the arrow keys  and

press <RETURN>.

The menu named Radii of Taper will now appear.

Radii of Taper

This menu is for entering or changing element radii for tapering

elements.

You can enter the radii for the reflector driver and the first

director only. The remaining directors are assumed to have the

same radii as the first.

The first data field for each element holds the radius of the

element section nearest to the boom and so on.

If all the elements have the same tapering radii you can speed up

the procedure by pressing <RETURN> when the cursor is placed at

the first data field of the second element. The previous elements

radii will be copied to the data fields of the current element.

Confirm the data with <OK> .

The Elementdata menu will now appear and you can now enter the

position of the elements. The Elementlengths and element radii

that will appear in the elementdata menu is the monotaper

calculated lengths and radii.(See Elementdata menu for more

information).

All the data have now been entered and you can select RUN to start

calculating.

Press <ALT> + R + <RETURN> and calculating will start.

An information box with the word "Working" will now appear on the

screen.

When the calculations are ready the sub-menu Data will appear with

a summary of the results.

LOAD

The Load option is for retrieving antenna files previously stored

on floppy disk or hard disk.

Press <ALT> + F and then "O". The menu for retrieving files will

now be shown.

All the accessible antenna files will be listed and you can select

the antenna file you wish by using the arrow keys. Press <RETURN>

and the antenna file will be loaded. You can also click with the

mouse on the antenna file you wish to load.

When the antenna file has been loaded the sub-menu Data will

appear showing a summary of the antenna data.

SAVE

To save an antenna file to disk select the Save option from the

Files menu.

The program will suggest a file name for the antenna. To accept

this name just press <RETURN> and the file will be saved.

If you want to give the file a different name press the arrow UP

key and change the name. Then press <RETURN> twice.

The file name may consist of up to 8 characters.

If the file name you have chosen is not accepted or if it already

exists an error message will be shown and you will be able to

change the name.

SAVE NEC

You can now save the antenna files from Yagi Analysis in NEC format.

If you have NEC you can  compute the antenna with NEC and then

use F6 in the pattern diagram to compare  results from Yagi

Analysis with NEC.

PRINT

The Print option can also be accessed from the Files menu and is

for printing the antenna data after calculation.

PLOT

If you want to plot the radiation pattern on an HP-7475 plotter

use the Plot option from the Files menu.

After selecting plot from the menu you will be asked if you want

to rotate the diagram 90 degrees. On certain plotters this

operation is necessary for the diagram to be placed correctly on

the paper.

The following communications protocol is used:

BAUDE RATE: 9600

DATA BITS: 7

STOP BITS: 2

PARITY: EVEN

CABLE: Standard plotter cable.

EXIT

If you want to exit Yagi Analysis select Exit from the Files menu.

EDIT MENU

Elementdata

The first sub-menu under Edit is Elementdata

This menu is for changing Position, length and radii for all

elements .

When Tapering element is chosen this menu only is for changing

element position.

(See elementdata menu for more information)

Length of Taper

This sub-menu is only shown when element taper is chosen, and

is for changing the taper lengths of all elements.

(See Length of taper above for more information)

Radii of taper

This sub-menu is only shown when element taper is chosen, and

is for changing the taper Radii of all elements.

(See Radii of taper above for more information)

Frequency

This menu is for changing start and step frequencies and the

number of frequencies to be calculated.

Enter the starting frequency in the first data field.

Enter the frequency step in the second data field. This is the

spacing between each frequency to be calculated.

Enter the number of frequencies to be analysed in the last data

field. The largest number allowed is 10.

Director number

If you want to lengthen or shorten an antenna select Director

number from the Edit menu.

The data field containing the current number will be shown. Enter

the new value and press <RETURN> twice.

If you increase the number of directors remember to also enter the

new element lengths and positions or spacing.

Boom corr

This sub-menu is for compensating element lengths for the diameter

of the antenna boom. The compensation will differ according to how

the elements are mounted.

Enter the diameter of the boom in the first data field. After

pressing <RETURN> the compensation based on a diagram after

measurement of DL6WU will be shown in the last data field.

This compensation is applicable if the elements are mounted

through the boom. If the current construction differs from this

you can press <TAB> twice to access <Help>. Press <RETURN> and you

will see a diagram that shows how boom compensation decreases as

the distance between the elements and the centre of the boom

increases. Select the mounting factor that is applicable to your

element mounting and press <RETURN>.

You will now be returned to the Boom corr menu. As the diameter of

the boom was not confirmed before selecting <Help> this must be

entered again.

Enter the mounting factor in the second data field and confirm the

data by pressing <TAB> until the cursor is on <OK> and press

<RETURN>.

The elements will now be lengthened by the boom corr factor and by

selecting the Print option from the Files menu you will obtain a

print-out of the new element lengths.

If you want to analyse an existing antenna you must remove the

boom compensation from the element lengths.

Measure the element lengths and enter these in the Element length

menu. Then select Boom corr and enter boom diameter and mounting

factor. Now place a minus in front of the boom corr factor.

The element lengths will now be decreased by boom corr and you can

start analysing the antenna.

RUN MENU

To calculate an antenna after entering or changing the antenna

data use RUN.

The program will then check for faulty data and if everything is

correct start to calculate. "Working" will be shown on the screen

while calculating is in progress.

When calculating is finished the sub-menu "Data" will appear

showing a summary of the results.

RESULTS MENU

Use this menu to view the results of a calculation.

Data

This is the first sub-menu and shows a summary of the results.

Polar E-pattern, H-pattern

These options are for viewing the radiation pattern in polar form.

Rectangular E-pattern, H-pattern

These options are for viewing the radiation pattern in rectangular

form.

If you have calculated several frequencies when selecting polar or

rectangular you will first see a sub-menu where you can select

which frequency you want to view.

Compare Nec

If you have NEC and have computed your antenna with NEC,

you can compare the radiation pattern from your antenna with the

pattern calculated with NEC.

Compare freq., ant.

You can compare the radiation pattern from a different antenna or

a different frequency (if more than one frequency has been

calculated).

By pressing F7 you can select another antenna and by pressing F8

another frequency.

The patterns will be superimposed on each other for a direct

comparison.

Screen dump

You can make a screen dump to a printer by pressing F9. If you are

using CGA graphics you must press Prt Sc.

Result-Graf

This option is for viewing Gain, F/B and VSWR as a function of

frequency.

After selecting this option you will come to a menu where you can

select which the reference frequency for VSWR .

At this frequency VSWR will be 1:1.

Press <ESC> to exit from the diagram.

Current-Graf

This function is for viewing the magnitude of the element current

with the element with the greatest magnitude as reference.

You can also compare the current from a different antenna with the

same number of elements or from a different frequency.

Current-Phase  (Graph)

This function is for viewing the phase of the element current

relative to the driver phase.

The distance in degrees between the driver and the other elements

has been added to the actual current phase of the elements to

give the real phase in the wave front.

You can also compare the phase from a different antenna with the

same number of elements or from a different frequency.

Current

Current shows the element currents and their phase position and

magnitude.

If you want to view the real element currents and their phase

positions you can select this option.

The currents are based on a supply voltage of 1 volt.

Showing Noise Temperature (G/T)

Shows Noise Temperature and G/T calculated with the OPTIONS

Noise Temperature (G/T). (See OPTIONS below for more information)

OPTIONS MENU

Dimension unit

This option is for switching units between millimetres, inches and

wavelengths.

If you have entered data in one unit you can change to another

unit with this option.

The unit you have selected will be stored in Grafik.fil.

Reflector type

This option is for changing the reflector configuration.

You can switch between single, double, triple, and triangular

reflectors.

The selected reflector configuration is stored in Grafik.fil.

Driver type

Here, you can select between straight or folded dipole.

If you select folded dipole you will then be able to enter the

length of the dipole and the spacing between the two dipole

elements.

If the calculation is to be correct it is important that the

dipole be mounted symmetrically on the boom.

Element pos

This option allows you to select the form in which the element

spacing is to be entered.

If you select "Distance between" you will enter the spacing

between two adjacent elements.

If you select "x-values" you will enter the real position of the

element on the boom with reference to the reflector which is "0".

Change Colors

This option allows you to select between three color settings:

Color, Mono, and LCD.

Wire Conductivity

To be able to calculate how efficient an antenna is, it is

important that the correct conductivity for the element material

is entered.

This option allows you to change the conductivity of the material

you are using.

The default value is 2.88E+07 which corresponds to 4104 and 6063-

T6 aluminium.

The <HELP> option will give you a table of the conductivity of

different alloys.

Ground/Stacking

If the antenna height and stacking distance are zero, the Yagi

is modeled in free space. Set the height to a nonzero value to

model the Yagi over a perfectly-conducting groundplane.

The antenna is always oriented for horizontal polarization

over ground. The elevation angle is that at which the E-Plane

patterns are computed. To model a stacked pair of Yagis in free

space, set the stacking distance to a nonzero value. This option

takes mutal impedances into account when modelling stacked Yagis

or a single Yagi over ground. You can't use more than 44 element

in this option. If you want to stack more than two antennas, or

use more than 44 elements you must use the stacking option under

RESULTS.

Stacking

Use this menu if you want to stack two or more antennas.

The spacing in H- and E-plane respectively can be entered here.

The first two data fields are for the number of antennas in the H-

and E-plane respectively, while the two last data fields are for

the spacing in each respective plane.

This option cannot calculate the coupling between the antennas, as

this would demand a very large memory and take too long when

stacking large antennas.

It is assumed that the coupling between antennas is negligible.

When stacking in the E-plane (horizontally) this assumption is

almost true even when the antennas are relatively close to each

other. However, when stacking in the H-plane the spacing must be

rather large to completely neglect the coupling between them. The

shortest spacing that can be used is one wavelength.

However, the coupling between antennas does not have too great an

effect on the radiation pattern, so this will still show the real

situation. The increase in gain though will always be 3 dB each

time the number of antennas is doubled in respective plane, which

is not always the case in reality.

For this option it is the radiation pattern that is of greatest

interest, not the gain.

Noise Temperature (G/T)

The effective noise temperature of antennas which "see" different

radiation sources with their pattern is a very important property,

because it limits the achievable system temperature of the

complete reciving system. This property can be measured by indirect

methods only. Noise temperature is dependent on the radiation pickup

of the main lobe in comparison to the pickup by the sidelobes.

For finite conductivity of the elements the resulting losses will

result in additional thermal noise.

The derived measure of G/T-ratio is a very suitable quality feature

to compare different Yagi constructions and different stacking schemes

for their value for EME-applications.Especially the latter criterion

has been subject of many, but mostly wrong, speculations of amateurs

in the EME-community.

The assumed elevation angle of the simulated Yagi  is 30 degrees.

The default temperature of sky is 15 K

The default temperature of earth is 290 K

Reference: DJ9BV "Effective Noise Temperature of 4-Yagi_Arrays for

432 MHz EME",  Dubus magazin 4/1987

YAGI ANLALYSIS FILE FORMAT

The file format is changed since previous version.

The extension is still .ANT and the file contain both input and

output data.

The first line is always ANTENNASPEC

Here is an example of a 15el Yagi

ANTENNASPEC

15144              ' Antenna Name

UNIT= mm           ' Dimension Unit can bee mm, Inch ,Wavel.

144 MHz            ' The saving frequency ( E- and H-Pattern)

143.5 MHz          ' Start frequency with swept antennas

145.5 MHz          ' Stop frequency  with swept antennas

REFLECTORTYPE= S   ' Reflector Type (S, D, T, R) see Options.

Directors = 13     ' Number of directors

Tapering=          ' If Tapering is chosen this line will be Yes

Tap = 0            ' Number of Taper

L1=278             ' L1 - L3 is distance between reflectors

L2= 0              ' when more than one reflector is used

L3= 0              ' Use <HELP> in Elementdata menu for more info.

L4= 0

Pos      ELLENGTH      RADIUS      ' These lines contain element

0        1092          3          ' position, lengths and radii

278      973           6

501      963           3

789      923           3

1303     913           3

1817     913           3

2331     894           3

2845     894           3

3359     874           3

3873     874           3

4387     854           3

4901     854           3

5415     834           3

5929     834           3

6443     814           3

COND= 2.88+07                  ' Wire Conductivity see Options

BOOMCORR = 0                   ' Boom Corrections

FOLDEDDIAMETER= 0              ' If folded dipol is used

DRIVER=Singel                  ' Folded if folded dipol is used

STEP=5                         ' Number of swept frequency

FREQUENCYSTEP= .5 MHz          '

***OUTPUTDATA***

DEG  E-PLANE                   ' Output data will contain

1         0                    ' 360 lines with E- and H-Plane

.                              ' pattern data and some other data

.                              ' used with current computing

.

Frequency   Gain     F/B      Re imp   Im imp    3-dBE    3-dBH

143.5        15.03    22.98   19.98    -3.45     16.62    17.94

144          15.05    22.13   19.3     -.98      16.47    17.75

144.5        15.05    21.38   18.43     1.5      16.32    17.58

145          15.02    20.74   17.32     4.11     16.2     17.43

145.5        14.95    20.14   15.95     6.99     16.1     17.32

E-PLANE= 1              ' used with stacking antenna

H-PLANE= 1              ' number of antenna stacked in each plane

GAIN= 15.05 dBI         ' Antenna Gain with the saved pattern

Efficency=98.7          ' Efficency in percent

This new file format  can't be used with the older version of

Yagi Analysis. So if you have an older version  you may take a

backup of your old .ANT files.

This version can read the older versions files but it saves them in

the new file format.